BioSync: Fingerprint Attendance System

IoT-Based Attendance Log Portal with Remote Handshake and Web Console

# 1. System Architecture & Workflow

The BioSync system features a real-time, bi-directional communication bridge between an ESP32 IoT node and an administrative React web application, synchronized via the Firebase Cloud.

Figure 1: BioSync System Architecture & Data Flow

# 2. Key System Features

Glassmorphic Admin UI: High-fidelity, modern dark design with real-time responsive feeds.

Remote Enrollment Wizard: Trigger fingerprint registration directly from the browser with a guided steps modal.

Remote Deletion Handshake: Remove database profiles and instruct the ESP32 to delete the template model from sensor memory.

Live Debug Terminal Console: Pipes system boot logs, scanning outputs, and network statuses into the web dashboard.

Background Settings Sync: Change confidence thresholds, buzzer beeps duration, and volume settings from the dashboard. The ESP32 pulls these settings live without reboots.

Diagnostic Logs & Exports: Track working days, attendance percentages, and download compiled attendance logs as CSV spreadsheets.

# 3. Hardware Wiring Reference

The system interfaces the ESP32 micro-controller board with the R307 optical fingerprint scanner, an active buzzer, a status indicator LED, and a tactile push button for local triggers.

Figure 2: ESP32 Hardware Wiring Schematic

## Pin-Out Connections Table

| Component | Component Pin | ESP32 GPIO | Connection Notes |
| --- | --- | --- | --- |
| R307 Sensor | VCC | 3.3V / 5V | Power supply pin |
| R307 Sensor | GND | GND | Ground pin |
| R307 Sensor | TX | RX2 (GPIO 16) | UART Serial Transmit |
| R307 Sensor | RX | TX2 (GPIO 17) | UART Serial Receive |
| Status LED | Anode (+) | GPIO 2 | Connected via 220-ohm resistor |
| Status LED | Cathode (-) | GND | Ground leg |
| Buzzer | Positive (+) | GPIO 4 | Active buzzer feed |
| Buzzer | Negative (-) | GND | Ground leg |
| Push Button | Terminal A | GPIO 5 | Triggers manual enrollment |
| Push Button | Terminal B | GND | Ground return |